# MATERIAL SAFETY DATA SHEET (MSDS)

Product Name:  TETOX aerosol insecticide

Date of Issue:     21st October 2020

## 1. PRODUCT AND COMPANY IDENTIFICATION

Product Identification: This product is a pressurized aerosol insecticide intended for household pest control.

Manufacturer:  Jordan Industrial Petrochemical Co. Ltd., Amman – Jordan,

Tel: +962-6-4611306, Email: jipco@nol.com.jo, website: www.jipco.jo

Registration:     Officially registered in Jordan under registration No. 757

Recommended Use:

- Control of flying and crawling insects in indoor and outdoor settings.

- Restricted to use as directed on the label.

- Not intended for agricultural or food-related pest control.

## 2. HAZARD(S) IDENTIFICATION

Classification:

- Flammability: Aerosol container with a flammable propellant.

- Health Hazards:

  - Excess inhalation of aerosol mist may cause respiratory irritation.

  - Long skin and eye contact may result in mild irritation.

  - Ingestion is not intended and can be harmful.

Label Elements:

- Hazard Pictograms:

  - Flame (flammable)

  - Pressurized container (cylindrical aerosol)

  - Harmful to Fish (toxic to fish)

- Signal Word: Warning

Precautionary Statements:

- Prevention:

  - Keep away from heat, sparks, open flames, and hot surfaces. No smoking.

  - Use only in a well-ventilated area.

  - Avoid contact with skin and eyes.

- Response:

  - In case of fire: Use appropriate extinguisher for chemical fires (foam, dry chemical, or CO₂).

  - In case of accidental exposure: Follow first aid measures (see Section 4).

## 3. COMPOSITION / INFORMATION ON INGREDIENTS

| Component | Chemical Name | Concentration | CAS Number |
| --- | --- | --- | --- |
| Active | Tetramethrin | 0.10 % | 7696-12-0 |
| Active | Cypermethrin | 0.30% | 52315-07-8 |
| Other | Inert Ingredients | 99.6% | Proprietary/Non-hazardous components |

Note: The remaining formulation consists of propellants and other inerts that are not classified as hazardous.

## 4. FIRST-AID MEASURES

Inhalation:

- Move the affected person to fresh air immediately.

- If breathing is difficult, administer oxygen and seek medical attention immediately.

Skin Contact:

- Remove contaminated clothing immediately.

- Wash skin thoroughly with soap and water.

- If irritation or redness persists, consult a physician.

Eye Contact:

- Rinse the affected eyes with plenty of water for at least 15 minutes.

- Remove contact lenses, if present.

- Seek medical attention if irritation continues.

Ingestion:

- Do not induce vomiting unless directed by doctor or medical personnel.

- Rinse mouth thoroughly with water.

- Obtain immediate medical care at hospital, show the insecticide contents on its label.

## 5. FIRE-FIGHTING MEASURES

Suitable Media:

- Use carbon dioxide, dry chemical, or foam extinguishers.

Special Protective Equipment for Firefighters:

- Wear self-contained breathing apparatus (SCBA) and full protective gear to avoid inhalation of hazardous fumes.

- Maintain a safe distance and cool surrounding containers if safe to do so.

Firefighting Instructions:

- Cool surrounding containers with water spray, if safe.

- Do not apply water directly to the aerosol can.

## 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions:

- Evacuate the area if necessary.

- Wear protective gloves, goggles, and respiratory protection as needed.

Spill Cleanup Procedure:

- For small, contained spills, ventilate the area and clean up using an inert absorbent material (e.g., dry sand).

- For larger spills ( Tanks ), isolate the area, notify appropriate emergency services, and follow environmental disposal.

## 7. HANDLING AND STORAGE

Handling:

- Handle only in well-ventilated areas away from ignition sources.

- Do not puncture, incinerate, or expose the container to excessive heat.

Storage:

- Store in a cool, dry, well-ventilated place away from direct sunlight and sources of ignition.

- Secure storage area to prevent accidental damage or tampering.

- Keep away from incompatible materials such as strong oxidizers.

## 8. PHYSICAL AND CHEMICAL PROPERTIES

| Appearance: | Aerosol liquid packaged in a pressurized container. |
| --- | --- |
| Odor: | Characteristic insecticide odor; may be slightly pungent. |
| pH: | Not applicable (aerosol mixture). |
| Boiling Point: | Not applicable for pressurized aerosols. |
| Flash Point: | Not determined, container is flammable. |
| Vapor Pressure: | The propellant is volatile. |

## 9. STABILITY AND REACTIVITY

Stability:

- The product is stable under normal conditions of use and storage.

Conditions to Avoid:

- Elevated temperatures, direct sunlight, and sources of ignition.

- Exposure to incompatible materials (e.g., strong oxidizing agents).

Incompatible Materials:

- Strong acids and bases, oxidizing agents, and other reactive chemicals.

Hazardous Decomposition Products:

- In the event of fire or thermal decomposition, release of toxic and/or flammable gases may occur.

- Follow fire-fighting measures accordingly.

## 10. TOXICOLOGICAL INFORMATION

Tetramethrin 	LD50 (mg/kg) Oral	>5000	species Rat

LD50 (mg/kg) Dermal	>5000	species Rat

Cypermethrin	LD50 (mg/kg) Oral	>250		species Rat

Acute Toxicity:

- High-level exposure may cause central nervous system effects, skin and eye irritation, and respiratory discomfort.

Chronic Effects:

- Chronic long term exposure to synthetic pyrethroids like tetramethrin and permethrin may result in sensitization.

- Although the concentrations here are low, repeated or prolonged exposure should be minimized.

- These compounds are not classified as human carcinogens when used according to label directions.

## Antidote and Treatment of Intoxication

- There is no antidote for permethrin or tetramethrin poisoning.

- Treatment consists in preventing further exposure together with supportive and symptomatic measures.

- In case of dermal exposure rinse the skin with abundant water and soft detergents.

ALWAYS SEEK MEDICAL HELP BY DOCTORS AT HOSPITALS in case of accidental ingestion.

## 11. ECOLOGICAL INFORMATION

Environmental Impact:

- The product is toxic to aquatic organisms; use caution to prevent environmental contamination.

- May also be harmful to beneficial insects like bees.

Persistence:

- Active ingredients may degrade under environmental conditions;

Biodegradability:

- Data on biodegradability is limited; proper disposal procedures should be followed to mitigate potential ecological impacts.

## 12. DISPOSAL CONSIDERATIONS

Disposal Recommendations:

- Waste disposal should be conducted in accordance with local, regional, and national environmental regulations.

Packaging Disposal:

- It may be necessary to depressurize the container under approved procedures, before disposal.

## 13. TRANSPORT INFORMATION

UN Number:

- Typically, aerosol insecticides fall under UN1950 (for aerosols) or related classifications depending on the propellant used.

Proper Shipping Name:

- Flammable Aerosol, Insecticide.

Hazard Class:

- Class 2 (Flammable Gas/Liquid) as determined by the overall product properties.

## 14. REGULATORY INFORMATION

Regulatory Status:

- The product should meet labeling and usage requirements of regional pesticide regulation agencies.

## 15. OTHER INFORMATION

Disclaimer:

The information provided in this MSDS is based on our current knowledge and is believed to be accurate.

This MSDS does not constitute a warranty of any kind what so ever.

It is the responsibility of the end user to ensure that the product is used and handled in safety and in compliance

with its label directions and the applicable local regulations.